# Non-Intrusive Estimation of Soot and Temperature Fields from Flame Images Using CNN and Synthetic Data

Abstract

This work presents a novel deep learning approach for non-intrusive, real-time estimation of soot volume fraction and temperature fields in axisymmetric laminar flames using RGB images alone. Traditional combustion diagnostic techniques are often intrusive, costly, or disruptive to flame structure, and unsuitable for continuous monitoring. To address this, we propose a physics-guided synthetic data generation pipeline that can produce high-fidelity flame images and corresponding ground-truth fields. These synthetic samples are used to train a deep learning convolutional neural network (CNN) based on an encoder-decoder architecture. The model performs pixel-wise regression to extract physical properties directly from flame images. The model achieves high accuracy and generalization, as demonstrated by low error metrics, highlighting the potential for real-time, non-intrusive, continuous, cost-effective monitoring of combustion processes with applications in research, scientific and industrial applications, power generation, and environmental monitoring systems.

1. Introduction

1.1 Background and Motivation

Understanding soot formation and temperature distribution is essential in combustion engineering, as these properties directly influence combustion efficiency, pollutant emissions, and the fundamental study of combustion physics [1] [2]. For co-flowing laminar ethylene/air diffusion flames- typical canonical flames for fundamental research- quantitative measurement of soot volume fraction (Fv) and temperature (T) provides critical insight into flame behaviour and pollutant formation mechanisms [3] [4].

Traditional diagnostic techniques for T and Fv face practical limitations. Thermocouples, for example, provide only point-wise temperature measurement and are susceptible to significant errors (up to 200 °C in sooty flames) due to radiative heat losses and soot deposition [2]. Infrared thermography and pyrometry can provide non-contact temperature measurements, but flame emissivity (particularly that of soot) is often unknown or highly variable, complicating absolute temperature estimation and requiring careful calibration, often by means of narrow-band ratio pyrometry [2] [3] [4]. Techniques such as laser-induced incandescence (LII) and planar LII (PLII) for soot quantification deliver high spatial resolution but demand expensive apparatus, extensive calibration, and are rarely feasible for continuous or in-situ industrial monitoring [1] [2] [3].

Moreover, detailed computational fluid dynamics (CFD) simulations that theoretically provide full-field data are computationally expensive, require significant expertise, and involve uncertainties due to combustion model complexity.

There is a clear need for methods that can extract quantitative combustion properties from easily acquired RGB flame images. Experienced human operators can often assess "flame health" visually, attesting to the richness of information in the flame's appearance; however, extracting accurate, spatially resolved physical data from such visual cues has historically remained elusive until the rise of modern computer vision and deep learning [2].

1.2 Deep Learning in Engineering Applications in Physical Field Estimation

Deep learning, and especially convolutional neural networks (CNNs), have revolutionized image analysis across many engineering fields, facilitating non-intrusive diagnostics that can learn complex spatial patterns inaccessible to conventional algorithms [2]. Encoder-decoder architectures, including fully convolutional networks and variants designed for dense (pixel-wise) prediction, are particularly well-suited for regression of underlying physical properties from images [5].

In this framework, the encoder progressively reduces spatial resolution while extracting hierarchical features, and the decoder reconstructs high-resolution output maps- enabling direct inference of fields like temperature and soot volume fraction from every pixel in an input image. Such architectures have been successfully tested for flame temperature mapping, soot estimation, and other physical property reconstruction tasks [3] [6]. The dense, end-to-end nature of encoder-decoder networks has rapidly expanded their adoption in combustion diagnostics, medical imaging, remote sensing, and scientific surrogate modelling.

1.3 Synthetic Data Motivation and Generation

A persistent challenge is the limited availability of pixel-wise ground truth data for flame temperature and soot volume fraction in real experiments: harsh, high-temperature environments, soot's strong absorption and scattering, and destructive or intrusive sampling make large, well-labelled datasets exceedingly hard to obtain [2] [6]. Intrusive measurements disturb the flame, while advanced optical methods require steady conditions and complex equipment; physical experiments are time-consuming, resource-intensive, and sometimes unreliable.

Synthetic data generation, where computational or analytical models produce physically plausible flames and related fields (T and Fv) and render them into RGB images, addresses these challenges. Modern synthetic datasets not only supply supervised training data for deep neural networks but also permit systematic variation of parameters and investigation of model robustness [6] [7]. Tools such as SootImage enable the simulation and rendering of realistic RGB flame images from simulated temperature and soot fields, providing rapid and repeatable sample generation [8].

Such synthetic datasets have been shown capable of training models that generalize—even to real experimental images—thus overcoming the major bottlenecks associated with data acquisition in combustion diagnostics [6].

1.4 Research Objectives and Contributions

This study addresses the fundamental challenge of non-intrusive combustion diagnostics by developing a computer vision system capable of estimating Soot Volume Fraction and temperature fields from standard RGB flame images. The key contributions of this work include:

- Novel Deep Learning Architecture: Development of a specialized CNN encoder-decoder network optimized for the estimation of several combustion properties

- Synthetic Data Generation: Creation of a comprehensive synthetic dataset with flame RGB image and GT flame properties

- High Accuracy Performance: Achievement of very low prediction errors

- Practical Implementation: Demonstration of real-time capability suitable for scientific and industrial applications

2. Methods

2.1 Synthetic Data Generation

Figure 1: Synthetic Data Generation Flow

The foundation of our approach lies in generating physically near-realistic synthetic flame data (co-flowing, laminar ethylene/air diffusion axisymmetric flame), building upon established combustion modelling principles. Traditional computational fluid dynamics (CFD) simulations [9] are computationally expensive and time-consuming for generating large datasets required for deep learning training.

co-flowing, laminar ethylene/air diffusion axisymmetric flame is a specific type of controlled combustion system used commonly in experimental combustion studies. Key features of such flames are that both the fuel (ethylene) and the oxidizer (air) flow in the same direction, usually vertically upward and smoothly, with no turbulence. The flame is also symmetrical around a central vertical axis.
Soot volume fraction and temperature are two fundamental properties that describe the state and behavior of a flame. They are particularly important when studying diffusion flames, such as co-flowing ethylene/air flames, where soot formation is significant. 
Soot volume fraction (denoted as Fv) represents the volume of soot particles per unit volume of the flame (ppm). Physically speaking, it means how much soot is present in a specific area of the flame. Soot affects flame luminosity, radiative heat transfer, and pollutant emissions. High Fv indicates inefficient combustion and harmful emissions. Fv can be measured or estimated from RGB flame color or luminosity patterns.
The temperature (denoted as T) represents the temperature in Kelvin at each point of the flame. Temperature varies across the flame and controls reaction rates, species formation, and soot kinetics. There are various temperature measuring techniques, some intrusive such as thermocouples and others such as optical pyrometry. It can also be predicted from RGB flame image.
The relationship between these two properties isn’t linear. High temperatures promote soot oxidation, reducing soot levels. Therefore researches often present Fv peaking in cooler regions. Understanding both properties together is key to designing cleaner, more efficient combustion systems.

2.1.1 Physical Model Development

Our synthetic data generation algorithm incorporates insights from validated numerical algorithms for modelling sooting laminar coflow diffusion flames [9] [10]. Chernov et al. [10] investigated the mechanisms of soot formation in partially premixed flames, providing a physical understanding that guided our parameter selection.

As we are discussing co-flowing, laminar ethylene/air diffusion axisymmetric flames- the structure of such flames is a wide base at the bottom, growing narrow to the flame’s tip. In the middle, we have a meaningful height – a cross section where the soot volume fraction is the highest (hmaxFv), making the information from this cross section particularly valuable and meaningful. Since the flame is axisymmetric, it is enough to analyze half of the flame to conclude about the whole.

The algorithm creates near-realistic flame profiles by defining key flame characteristics: flame heights, soot volume fraction profiles and temperature profiles.
Two critical flame heights are randomly generated:

- hmaxFv – this is a height that holds the maximum Fv (Soot Volume Fraction) value of the flame (random(200-400))

- htop - the top (currently, it isn’t the tip of the flame, but it is the top height of the flames we produce) (random(200-400) + hmaxFv)

- When relating to the base of the flame, height = 0

Figure 2: Fv cross-sectional profiles

Figure 3: Fv contour

A cross-sectional physically accurate profile is generated for Fv and T in each of these three heights. 
For soot volume fraction profiles, three representative cross-sectional profiles are generated at the 3 picked flame heights (Figure 2): fbase (Base profile with maximum flame width), fmaxFv (Profile at maximum soot concentration height) and ftop (Top profile with minimum flame width). Based on them, I create the flame’s Fv contour (Figure 3). The Fv contour will be the flame’s edges I will regard to when dealing with both the Soot Volume Fraction and the Temperature properties.

Figure 4: T cross-sectional profiles

Figure 5: T contour

Similarly, three physically accurate temperature profiles are generated: tbase, tmaxFv, ttop at corresponding heights (Figure 5). The T vectors are generated vertically (Figure 4), so I transpose them before continuing.
Flame’s T contour is different than the flame’s Fv contour. It is generally wider, and there is no rule as to which section (ttop, tmaxFv, or tbase) is wider. In any case, populating the T matrix is done according to the flame’s Fv contour.

As mentioned, generating the data is done with a few random picks. Therefore, in order to provide data physically and mathematically as accurately as possible, a few restrictions and conditions were defined before the data generated is accepted for the next steps.
Once the two heights (htop, hmaxFv), three Fv vectors (fbase, fmaxFv, ftop) and three T vectors (tbase, tmaxFv, ttop) are generated as detailed above, the data goes through several verification steps detailed below. If verification fails, the data is regenerated until all terms and restrictions are met:

- hmaxFv, htop, fbase, fmaxFv, ftop, tbase, tmaxFv, ttop set was not used before.

- len(fbase)>len(fmaxFv)>len(ftop).

- The max value of fmaxFV is greater than max value of fbase and greater than max value of ftop.

- All T vectors’ lengths must be greater than fbase length.

* There is no restriction as to which vector is longer between the T vectors.

2.1.2 Matrix Population Through Linear Interpolation

The complete flame property fields are constructed using linear interpolation between the generated profiles for any pixel location (x,y) within the flame boundary:

Soot Volume Fraction Matrix:

I’ve created a Fv matrix (matfv) of zeros with a shape slightly wider than the length of fbase and slightly higher than htop. Now, since I’ll attend only the cells within the Fv contour, values outside the contour remain 0.
Next I insert fbase, fmaxFv and ftop vectors at  y=0, y=hmaxFv and y=htop respectively. Cells on the Fv contour get the value of 0.1. At this point I got something like (Figure 3: Fv contour. Filling in the missing data inside the Fv contour boundaries is done by calculating a linear interpolation for each gap between the bottom vector and the top vector, depending on the weight calculated according to the Y position (1). This means when weight is 0, I’ll get values as bottom_vector[x]; when weight is 1, I’ll get values as top_vector [x]; and for values in between, I’ll get a blend. By doing this, I get smooth transitions from the base to the top as I move vertically (Figure 6).
As mentioned, this is done in both gaps- one between fbase and fmaxFv, and the other between fmaxFv and ftop.

Recall the structure of the flame- the bottom vector is longer than the top vector. This results with 2 optional cases I address within the Fv contour- x is within the range of the top vector or x is outside the range of top_vector (meaning top_vector[x] does not exist, and the current matrix value is 0). For the first case we will interpolate between bottom_vector[x] and  top_vector [x] (2) and for the latter- interpolate between base[x] and a small constant value of 0.1 (3).

| weight=yhmaxFv | (1) |
| --- | --- |
| 1-weight*bottom_vector[x]+weight*top_vector[x] |  |  | (2) |
| 1-weight*bottom_vector[x]+weight*0.1 |  |  | (3) |

Figure 6: Calculated Fv map from the three generated profiles. In blue- Fv contour.

Temperature Matrix:

A similar interpolation approach is applied to T (matt) (Figure 7). Reminder- according to Fv contour:

I’ve created a T matrix (matT) with 300 (units are in Kalvin, so this is the zero value)  with a shape slightly wider than the maximum lengthes between tbase, tmaxFv and ttop and slightly higher than htop. Now, since I’ll attend only the cells within the Fv contour, values outside the contour remain 300.
Next I insert tbase, tmaxFv and ttop vectors at  y=0, y=hmaxFv and y=htop respectively. Cells on the Fv contour get the value of 300. Filling in the missing data inside the Fv contour boundaries is done by calculating a linear interpolation for each gap between the bottom vector and the top vector, depending on the weight calculated according to the Y position (1). This means when weight is 0, I’ll get values as bottom_vector[x]; when weight is 1, I’ll get values as top_vector [x]; and for values in between, I’ll get a blend. By doing this, I get smooth transitions from the base to the top as I move vertically (Figure 7).

As mentioned, this is done in both gaps- one between tbase and tmaxFv, and the other between tmaxFv and ttop.

(4)

Recall the structure of the flame- (by definition) the shortest temperature vector is longer than fbase (the longest Fv vector) and because we are interested in the Fv contour, this results with 2 optional cases - x is within the range of fbase vector or x is outside the range of fbase vector (meaning top_vector[x] does not exist, and the current matrix value is 300). For the first case we will interpolate between bottom_vector[x] and  top_vector [x] (2) and for the latter- interpolate between top[x] and a small constant value of 300 (4).

| 1-weight*300+weight*top_vector[x] |  |
| --- | --- |

Figure 7:  Calculated T map from the three generated profiles according to Fv contour. In blue- T contour.

2.1.3 RGB Image Generation

Synthetic RGB flame images are generated using the SootImage tool implemented in MATLAB [8]. SootImage was created to obtain an image of the flame based on the computed properties  soot volume fraction and temperature, to compare it to an actual image of the flame being studied, hence validating the properties computations. It recreates the image by integrating the soot thermal radiation over a line-of-sight. Soot is assumed to emit as grey body. The integrated radiation is passed through the colour filters of the camera and is translated to pixel values.

As the validity of the tool has already been proven, we utilized it for generating a co-flowing, laminar ethylene/air diffusion axisymmetric RGB flame image based on our synthetically calculated Fv and T matrices.

The process involves creating the mat file needed as input- a sootCalculation.mat  (holding matfv  as fv (soot volume fraction), matt clipped to matfv’s shape as T (temperature), a vector of size as the smaller side of the matrices with values from 0 in steps of 0.0662 (pixel size) as r (radial coordinates), a vector of size as the larger side of the matrices with values from 0 in steps of 0.0662 (pixel size) as z (axial coordinates)).
Inputting sootCalculation.mat to sootImage tool (MATLAB files) outputs *.mat files holding the flame’s RGB image matrices.

2.1.4 Outputs

For the DL model dataset, I need the following input and outputs per sample:

- Input- RGB flame image (extracted from CFDImage.mat)

Figure 8: JPG representation of CFDImage

Output- Soot Volume Fraction in every pixel of the flame (extracted from sootCalculation.mat)

- Output- Temperature in every pixel of the flame (extracted from sootCalculation.mat)

Using the processes and algorithms described above, I can create a dataset without the costly resources of a simulator.  Generating a sample takes approximately 30 seconds, varying in correspondence with the drawn size of the flame and times regenerated until conditions are met.

The synthetic data pipeline generates several files. Not all files are needed for building the DL’s dataset. The latter are used for analyzing the quality of the data generated.

2.2 Deep Learning CNN Model

Deep learning, particularly convolutional neural networks (CNNs), has become a powerful tool for pixel-wise regression tasks in scientific imaging domains. Encoder-decoder architectures, often inspired by the U-Net framework [11], are especially effective for such problems due to their ability to extract features and reconstruct dense spatial maps. These models are well-suited for tasks like estimating physical properties from visual data, where each pixel in the input image corresponds to a continuous target value. In the context of combustion diagnostics, CNN encoder-decoder models have been used to infer temperature and soot volume fraction directly from RGB flame images [12] [13], offering a non-intrusive, data-driven alternative to traditional techniques such as laser-induced incandescence (LII) and thermocouple measurements. Moreover, physics-informed synthetic data can be used to train such models effectively when real labelled datasets are limited or unavailable [14], enhancing their generalizability and physical fidelity.

This paper presents a deep learning approach for pixel-wise regression of soot-related properties directly from RGB flame images. Specifically, I designed a CNN-based encoder-decoder-like architecture that maps a flame RGB image to two heatmaps representing physical fields – Soot Volume Fraction (Fv) and temperature (T).

Key Contributions:

- A deep encoder-decoder architecture for high-resolution per-pixel prediction of physical quantities from flame imagery

- A supervised learning pipeline based on synthetic data, providing maps of Fv and T

- A quantitative and qualitative evaluation of model predictions with visualizations

Figure 9: CNN Encoder-Decoder Design.

2.2.1 Deep Learning Architecture

Figure 10: CNN Encoder-Decoder Blocks

The CNN Encoder-Decoder model includes (Figure 10) 3 modules- Encoder with 3 initial convolution layers + 4 residual blocks (with skip connections), Decoder with 4 residual blocks and Final module with 2D convolution followed by a Sigmoid activation function.

The detailed architecture (Figure 11) demonstrates a U-Net like CNN with a contracting path (encoder) and an expanding path (decoder). 
The encoder begins with an initial convolutional block that reduces the spatial resolution of the input RGB image and increases the feature depth. Specifically, a 7×7 convolution with 64 output channels, a stride of 2, and padding of 3 is applied, followed by batch normalization and ReLU activation. This forms the initial layer.
Following the initial block, the encoder comprises four sequential residual blocks. Each residual block consists of two 3×3 convolutions with batch normalization and ReLU, along with a skip connection that adds the input directly to the output. These blocks progressively reduce the spatial dimensions while increasing the channel depth:

- Block 1: 64 → 128 channels, with stride 2

- Block 2: 128 → 256 channels, with stride 2

- Block 3: 256 → 512 channels, with stride 2

- Block 4: 512 → 512 channels, with stride 1

The output of each block (including the initial convolutional layer) is stored as a feature map, which is later reused in the decoder for skip connections.

The decoder reconstructs the high-resolution feature map using four upsampling blocks. Each block performs a 2×2 transposed convolution (deconvolution) to double the spatial resolution, followed by batch normalization, ReLU activation, and dropout for regularization. The upsampling sequence is as follows:

- 512 → 512 channels (dropout = 0.2)

- 512 → 256 channels (dropout = 0.3)

- 256 → 128 channels (dropout = 0.2)

- 128 → 64 channels (dropout = 0.1)

Each upsampled feature map is fused with the corresponding encoder feature via an additive skip connection. If the spatial dimensions do not match, the encoder features are interpolated using bilinear upsampling to ensure compatibility.

After the decoder, a final 1×1 convolution is applied. A sigmoid activation ensures that output values are in the normalized [0, 1] range. If necessary, the final output is resized using bilinear interpolation to match the original image dimensions.

Figure 11: CNN Encoder-Decoder Architecture

Before running the training process, the data is extracted from *.mat files, analyzed to get global parameters for best normalizations and the dataset is created.

Each epoch in the training runs train-validation sets. Each time the validation loss improves- the model is saved as best model. Training ends when number of epochs exceeds or epoch patience exceeds, which ever comes first. Epoch patience is set to 10 epochs, meaning if the validation loss does not improve upon 10 consecutive epochs- early stop training. At the end of training- the best model is loaded, and a test set is run.

2.2.2 Network Training Configuration

Loss Function: Combined mean squared error (MSE, (5) for each of Fv, T outputs with a weight of 50% each.

(5)

Optimization Strategy:

- Adam optimizer with learning rate 0.0001

- Batch size: 12 samples (optimized for available GPU memory)

- Maximum epochs: 300 with early stopping patience: 10 epochs (after 10 epochs without improvement → stop training and continue to the test and plots)

Learning Rate Scheduling:  ReduceLROnPlateau scheduler monitors validation loss- Reduce lr (Learning Rate) in case of plateau in validation loss (no improvements after patience period (3 epochs), then learning rate will be reduced (6).

| lrnew=lr*factor, factor=0.3 | (6) |
| --- | --- |

Adaptive learning prevents overfitting and improves convergence

2.2.3 Dataset Creation and Data Manipulations

To train and evaluate the model, a large-scale synthetic dataset of flame images was generated. The dataset was curated by extracting valid image ranges from the original *.mat simulation files, explicitly removing any instances containing invalid values—those exceeding 20,000 or falling below zero. All input RGB images and corresponding ground truth outputs (temperature and soot volume fraction) were normalized based on global dataset-wide statistics. Specifically, the global minimum and maximum values across the entire dataset were computed and used for normalization.

As part of preprocessing, several critical transformations were applied. First, all images were vertically flipped (if needed) to enforce a consistent flame orientation, ensuring the wider flame base appears at the top across samples. Then, small target values were thresholded: temperature values below 1000 K and soot volume fractions (Fv) below 0.01 ppm were reset to zero. After thresholding, normalization was performed using the following dataset-specific value ranges globally computed as mentioned above: image intensity values were scaled from [0, 19,941.03] to the range [0, 1]; temperature values were normalized from [299 K, 2828 K] to [0, 1]; and soot volume fraction values were normalized from [0, 11.2248 ppm] to [0, 1].

To ensure consistent dimensions during training, all images were padded to the maximum observed resolution in the dataset, resulting in a uniform shape of 808×213 pixels.

The full dataset contained 11,868 synthetic flame samples, which were divided into training, validation, and test sets using a 70-20-10 split. 
For evaluation purposes, a final postprocessing step was applied only to test data: values that had previously been thresholded (i.e., below 0.01 for Fv or 1000 K for temperature) were again identified after normalization and reset to zero to ensure consistent error analysis.

2.2.4 Outputs

To best analyse and examine results, I output several plots and files. Some are plotted as part of the training process (validation samples) to provide snapshots within the epochs. Others are saved to offer analysis capabilities and quantify the results.

Heat maps of GT and prediction T, Fv are plotted for several samples, along with an error map giving a perspective where the problematic regions are.

Best model *.pth file is saved and overwritten in every validation loss improvement. Losses are plotted in a smoothed graph.

2.2.5 Inference and Single Tests

All training samples are padded to match the largest image in the dataset. However, when taking an unseen sample (out from the training dataset)- it might be larger. In this case, I first crop the marginal rows and columns that hold only zeros. If still to big- I crop from the right and the bottom (reminder- the bottom of the image is the tip of the flame due to the flip process).

When running an unseen sample, I can either test it (if I have the image and GTs) and analyze its loss and error maps or inference- (if I only have an image). In any case the model predicts the T and Fv and plots a heatmap of them.

3. Results

3.1 Training Quantitative Results

Table 1: Training quantitative info

| Average epoch | Train ~6 min, Validation ~1 min |
| --- | --- |
| Best model saved at epoch | 42 |
| Best validation loss | 0.00012012 |
| Test loss | 0.00012670 |

These low loss values significantly outperform traditional correlation-based methods and results.

Figure 12: CNN Encoder-Decoder smoothed Losses Plot. The red dashed line represents the test loss value.

| a. |  |
| --- | --- |
| b. |  |
| c. Figure 13: Visualize full error distributions (histogram and boxplot) for train set (a.), validation set (b.) and test set (c.). |  |

Several evaluation metrics were run on the training model’s results to analyze and evaluate the model:

MSE- Mean Square Error

| MSE= 1Ni=1N(yipred-yigt)2 | (7) |
| --- | --- |

MSE is calculated to highlight large mistakes. Tends to be higher than MAE.

MAE- Mean Absolute Error

The average of absolute differences between the model’s prediction and GT.

| MAE= 1Ni=1N¦yipred-yigt¦ | (8) |
| --- | --- |

MAE indicates (on average) how far off your predictions are. Less sensitive to outliers than MSE.

R2 score (coefficient of Determination)

| R2= i=1N(yigt-yipred)2i=1N(yigt-ygt)2 | (9) |
| --- | --- |
| ygt= 1Ni=1Nyigt | (10) |

R2 score is calculated by dividing the Sum of squared predicted error (model' s total error) with the total variance in the GT values (how far each true value is from the average) (9). ygt in the denominator is the mean of GT values (10).

R2<0 – model is worse than using just the mean.
R2>0 – model is better than the mean.
R2=1 – model is perfect.

Mean±std (Mean and Standard Deviation)

Mean: the average value.
std: the spread of the values around the mean.

| std= 1Ni=1N(xi-x)2 | (11) |
| --- | --- |

Mean±std provides a quick view of central tendency and variability (=where most samples “fall”).

All metrics were calculated twice: once on the whole map of Fv/T and once on the most meaningful row in the map, which is the index of the row (height) with the highest Fv value.

Table 2: Mean ± standard deviation of regression metrics for Soot Volume Fraction (Fv) and temperature (T). Errors are reported over the full flame map ("Full") and the most meaningful row ("Row") across training, validation, and test sets. MSE and MAE are shown in units x10⁻³.

| Set | Quantity | Region | MSE (x10-3) | MAE (x10-3) | R2 Score |
| --- | --- | --- | --- | --- | --- |
| Train | Fv | Full | 0.00002 ± 0.00001 | 0.00174 ± 0.00034 | 0.99797 ± 0.00102 |
|  |  | Row | 0.00006 ± 0.00010 | 0.00308 ± 0.00130 | 0.96503 ± 0.11792 |
|  | T | Full | 0.00014 ± 0.00012 | 0.00278 ± 0.00045 | 0.99691 ± 0.00271 |
|  |  | Row | 0.00065 ± 0.00090 | 0.00635 ± 0.00300 | 0.98885 ± 0.01655 |
| Validation | Fv | Full | 0.00002 ± 0.00001 | 0.00177 ± 0.00035 | 0.99789 ± 0.00122 |
|  |  | Row | 0.00007 ± 0.00011 | 0.00312 ± 0.00125 | 0.96613 ± 0.05033 |
|  | T | Full | 0.00022 ± 0.00023 | 0.00292 ± 0.00060 | 0.99518 ± 0.00495 |
|  |  | Row | 0.00092 ± 0.00125 | 0.00679 ± 0.00330 | 0.98434 ± 0.02282 |
| Test | Fv | Full | 0.00002 ± 0.00002 | 0.00139 ± 0.00040 | 0.99788 ± 0.00169 |
|  |  | Row | 0.00007 ± 0.00011 | 0.00290 ± 0.00137 | 0.96312 ± 0.06570 |
|  | T | Full | 0.00023 ± 0.00029 | 0.00163 ± 0.00078 | 0.99492 ± 0.00644 |
|  |  | Row | 0.00112 ± 0.00166 | 0.00590 ± 0.00412 | 0.98099 ± 0.02939 |

Figure 14: Extract per-sample Fv MAEs for training, validation, and test sets from full map metrics (top) and row-of-interest metrics (bottom). Each circle indicates a sample. The dotted line indicates the mean of each set.

Figure 15: Extract per-sample Fv MSEs for training, validation, and test sets from full map metrics (top) and row-of-interest metrics (bottom). Each circle indicates a sample. The dotted line indicates the mean of each set.

Figure 16: Fv Coefficient of determination (R²) per sample for training, validation, and test sets. High R² values across samples indicate strong generalization. Each circle indicates a sample.

| Figure 17: Extract per-sample T MAEs for training, validation, and test sets from full map metrics (top) and row-of-interest metrics (bottom). Each circle indicates a sample. The dotted line indicates the mean of each set. |  |
| --- | --- |
| Figure 18: Extract per-sample T MSEs for training, validation, and test sets from full map metrics (top) and row-of-interest metrics (bottom). Each circle indicates a sample. The dotted line indicates the mean of each set. |  |
| Figure 19: T Coefficient of determination (R²) per sample for training, validation, and test sets. High R² values across samples indicate strong generalization. Each circle indicates a sample. |  |

Based on R², MAE, and MSE metrics, our model is evaluated as highly accurate for both temperature and soot volume fraction and is performing very well.
R² Scores are above 0.96 — That's a strong indication the model fits the data very well.
MAE (≈ 0.002–0.009), Low error — This is a small deviation.
MSE Values < 0.001 — this is expected for small per-pixel differences.

a. b.

c. d. Figure 20: Relationship between mean absolute error (MAE) and mean squared error (MSE) per sample (log scale) in the different test sets. a. For full Fv map. b. For Fv most meaningful row. c. For full T map. d. For T most meaningful row. Points aligned diagonally imply consistent error scaling, while vertical gaps may indicate outlier-driven MSE.

Figure 21: Distribution of R² scores across training, validation, and test sets for Fv (left) and T (right). Each dot represents one sample. Violin plots summarize the distribution shape. High and consistent R² values indicate strong model generalization.

Figure 22: Distribution of mean absolute error (MAE) across training, validation, and test sets for Fv (left) and T (right). Tight clustering in the training set and modest spread in validation/test suggest low prediction error and generalization capability.

3.2 Training Qualitative Results

The model’s results can also be analyzed visually, using the plots saved during validation and test. By creating heatmaps and error maps – we can qualitatively assess what we saw quantitively in the previous chapter.

For several samples, I visualized the whole process- from original flame image input to the processed image, to the Fv, T prediction, GTs and error maps through the epochs (Figure 23: RGB input → prediction → error maps → ground truth. Qualitative results on representative validation sample #8813 through the epochs for temperature (T) (top) and Soot Volume Fraction (Fv) (bottom). The model’s predictions closely match the ground truth (GT), with low per-pixel error evident as presented in the error maps..

Another visualization is the scatter of prediction vs. GT (x=y is the perfect match) (Figure 24).

3.2.1 Validation Sample Results

Figure 23: RGB input → prediction → error maps → ground truth. Qualitative results on representative validation sample #8813 through the epochs for temperature (T) (top) and Soot Volume Fraction (Fv) (bottom). The model’s predictions closely match the ground truth (GT), with low per-pixel error evident as presented in the error maps.

Figure 24: Pixel-wise scatter plot of predicted vs. ground truth values for Fv (left) and T (right) in a validation sample. The diagonal line indicates perfect prediction (x=y). Points clustered near this line confirm high regression accuracy. T presents more outliers. This is due to area measurement capabilities. See the discussion chapter for an explanation.

Figure 25: RGB input → error maps → prediction → ground truth. Qualitative results on representative test sample #6535 for temperature (T) (top) and Soot Volume Fraction (Fv) (bottom).

Axial Position (z) [mm]

Pred

GT

T

Fv

6535 (test)

Loss: 0.00003379

3.2.2 Test sample results

Figure 26: Pixel-wise scatter plot of predicted vs. ground truth values for Fv (left) and T (right) in a test sample #6535. The diagonal line indicates perfect prediction (x=y). Points clustered near this line confirm high regression accuracy. T presents more outliers. This is due to area measurement capabilities. See the discussion chapter for an explanation.

Another view of this test sample results- 3 cross sections in different heights (flame’s edges and meaningful height- where Fv value is the largest), comparing the GT in green and the prediction in orange for each height (Figure 27).

Figure 27: Fv and T prediction (orange) vs. GT (green) in 3 heights- 30, 244 (meaningful height- where Fv value is the largest), 400 pixels. The more the green and correlated orange are aligned, the closer the prediction is to the GT.

3.3 Inference and Single Test

All training samples are padded to match the largest image in the dataset. However, when taking a sample outside of the training dataset or a real data sample, it might be larger. In this case, I first crop the marginal rows and columns that hold only zeros. If still too big- I crop from the right and the bottom (reminder: the bottom of the image is the tip of the flame due to the flip process).

When running a single sample:
If we have only the flame image: run in inference mode (predict only).
If we have the flame image and GTs: run in single test mode (predicts and calculates loss, plots results).

3.3.1 Single test on unseen synthetic sample (not in training dataset)

Running a single test on unseen images yielded an interesting result: While some samples showed a low error and appeared accurate in the meaningful height (Figure 28, Figure 29), others had an even lower error but exhibited a spot of inaccuracy in the

Pred

GT

T

Fv

10 (unseen)

Loss: 0.00012300

meaningful height (Figure 30, Figure 31).

Figure 28: RGB input → error maps → prediction → ground truth. Qualitative results on representative unseen synthetic data sample #10 for temperature (T) (top) and Soot Volume Fraction (Fv) (bottom).

a.

b. Figure 29: Fv and T prediction (orange) vs. GT (green) in 3 heights- 30, 383 meaningful height (where Fv value is the largest), 500 pixels. The more the green and correlated orange are aligned, the closer the prediction is to the GT.

Pred

GT

T

Fv

58 (unseen)

Loss: 0.00008343

Figure 30: RGB input → error maps → prediction → ground truth. Qualitative results on representative unseen synthetic data sample #58 for temperature (T) (top) and Soot Volume Fraction (Fv) (bottom). Notice that although the lower error, you can easily see the hiccup in the meaningful area (circled in green).

a.

b. Figure 31: T (a.) and Fv (b.) prediction (orange) vs. GT (green) in 3 heights- 30, 228 (meaningful height- where Fv value is the largest), 400 pixels. The more the green and correlated orange are aligned, the closer the prediction is to the GT. Although the prediction is quite accurate along with the GT values, notice the hiccup at meaningful height, both in T and Fv.

3.3.2 Single test on unseen real samples

Real data may be slightly different than synthetic data. Our model was trained on synthetic data only, so every real sample is unseen. Observing (Figure 32, Figure 33), you can see the output is different than the data the model was trained on. However, the cross sections show that in the meaningful height- where Fv value is the largest - the prediction is very accurate. The high loss can be explained due to a significant inaccuracy especially around the bottom of the flame, more significant in the temperature than in the Fv.

Pred

GT

T

Fv

00 (unseen real)

Loss: 0.04083942

Figure 32: RGB input → error maps → prediction → ground truth. Qualitative results on representative unseen real data sample #00 for temperature (T) (top) and Soot Volume Fraction (Fv) (bottom).

Figure 33: T and Fv prediction (orange) vs. GT (green) in 3 heights- 30, 506 (meaningful height- where Fv value is the largest), 800 pixels, for unseen real data sample #00.  The more the green and correlated orange are aligned, the closer the prediction is to the GT. The results are good in the meaningful height. Notice that in h=30 in the temperature graph below ~3.5 mm, we do not know how to measure the temperature, so we do not have an evaluation of the prediction in that domain.

3.3.3 Inference on real data

As mentioned, inference is a mode where we have the CFDImage.mat holding the RGB flame matrix as an input, but we do not have a GT output to compare to. Following are a couple of inference runs, done on real data (unseen) samples.

T

Fv

Figure 34: Input and prediction (Fv and T) in inference, on an unseen real data sample #0. As part of matching the training conditions, I Took out zeros from flame image matrix margins plus cropped matrix end rows to fit target shape: Padded/Cropped image from original shape: (979, 98, 3) to target shape: (3, 808, 213). The image was also flipped.

T

Fv

Figure 35: Input and prediction (Fv and T) in inference, on an unseen real data sample #4. Notice the flame was flipped to match our training conditions and was padded/cropped from original shape: (979, 98, 3) to target shape: (3, 808, 213).

4. Discussion

The results of this study demonstrate that a convolutional neural network trained entirely on synthetic data can accurately infer temperature and soot volume fraction fields from flame RGB images. Both qualitative and quantitative evaluations indicate strong model generalization and regression performance.

Figure 36: Demonstrating the outliers in prediction vs. GT for sample # 8813 in the temperature properties. Analysing the 4 points circled in orange, mapping them on the temperature heatmap shows they are positiond on the contour, which confirms the hypothesis that the serrated contour is the cause for these big gaps between predictions and GTs.

Figure 37: Temperature (T) jagged contour example

The model’s evaluation metrics were calculated for Fv and T and then combined in 1:1 ratio. From the results, we can conclude that T has more outliers than Fv. Carefully analysing them, we thought they are a direct result of the data structure: the contour of the T is jagged (pixeled) and does not gradient as Fv does (Figure 37). Therefore, there are large value distances between adjacent pixels. We’ll demonstrate it on the sample previously shown at (Figure 23): From T prediction vs. GT, we can see several outliers positioned far from the desired x=y line. Let’s examine the 4 points in the orange circle- first, we found the 4 indices of the points where GT is 0.5-0.6 and prediction is < 0.05. Mapping the indices on the GT and prediction heatmap confirmed our hypothesis- they are well placed on the jagged contour (Figure 36). 
By carefully examining each point and comparing its location in GT and prediction, we can see that the error may be big even though it’s tolerable (Figure 38). 
This also explains the MAE vs. MSE vertical gaps (Figure 20).

Figure 38: Zooming in on the 4 points proves the hypothesis that the jagged edge contour (large value distances between adjacent pixels) causes several big distances between prediction and GT values.

The model achieved high R² scores for both temperature and Soot Volume Fraction prediction across training, validation, and test sets, with low MAE and MSE values even on unseen samples.

One of the key enablers of this success was the carefully designed synthetic data generation pipeline. By encoding combustion-relevant physical structures into randomizable but constrained parameters, the resulting dataset maintained diverse without sacrificing realism. The use of SootImage for rendering ensured visual fidelity and consistency between the physical and image domains, facilitating effective learning.

The encoder-decoder architecture proved suitable for high-resolution, pixel-level prediction. It was able to preserve both fine local structures and global flame contours. The inclusion of dropout, skip connections, and adaptive learning rate contributed to training stability and reduced overfitting.

The row-based error analysis (at the location of maximum Fv value) provided additional insight into model behaviour in flame-core regions, where peak physical phenomena occur. Despite running on real flames with different structure or properties, the model maintained high accuracy in this meaningful region, highlighting its robustness. This was showcased as single-sample test and inference on real-world flame images and showed that the model, trained entirely on synthetic data, could extract physically meaningful estimations even when faced with real noise and domain shifts. This indicates that the synthetic training set sufficiently captures key flame features, enabling transfer to experimental data. However, performance on real data would likely benefit from domain adaptation techniques or fine-tuning with a small real dataset.

Future work could explore extending the model to other flame geometries (e.g., turbulent, 3D), integrating uncertainty estimation, and incorporating time-series consistency. Additionally, combining synthetic and experimental datasets in training may improve robustness and enable deployment in live combustion systems.

5. References

| [1] | Cruz JJ, Verdugo I, Gutiérrez-Cáceres N, Escudero F, Demarco R, Liu F, Yon J, Chen D and Fuentes A (2021) Soot Volume Fraction Measurements by Auto-Compensating Laser-Induced Incandescence in Diffusion Flames Generated by Ethylene Pool Fire. Front. Mech. Eng 7:744283. doi: 10.3389/fmech.2021.744283 |
| --- | --- |
| [2] | GUO, Haiqing, et al. A Digital Imaging Technique to Measure Temperatures and Soot Concentrations in Flames of Condensed Phase Fuels. United States. Department of Transportation. Federal Aviation Administration. William J. Hughes Technical Center, 2021 |
| [3] | Legros, Guillaume, et al. "Simultaneous soot temperature and volume fraction measurements in axis-symmetric flames by a two-dimensional modulated absorption/emission technique." Combustion and Flame 162.6 (2015): 2705-2719 |
| [4] | Das et al., "Two-dimensional soot volume fraction measurements in coflow diffusion flames using color ratio pyrometry," 2017 |
| [5] | Li Z, Wan N, Qian X. Measurement of Soot Concentration in Burner Diffusion Flames through Emission Spectroscopy with Particle Swarm Optimization. Sensors (Basel). 2024 Feb 17;24(4):1292. doi: 10.3390/s24041292. PMID: 38400449; PMCID: PMC10891556 |
| [6] | Toledo-Marin et al., "Deep learning-based surrogate modeling for spatiotemporal field regression with synthetic data," arXiv:2302.03786 |
| [7] | V. Chernov et al., "Numerical investigation of soot formation mechanisms in partially-premixed ethylene–air co-flow flames," Combustion and Flame, vol. 159, no. 9, 2012 |
| [8] | V. Chernov et al., "SootImage: An image recreation, post-processing validation procedure for sooting axisymmetric flames," Computer Physics Communications, vol. 300, 2024 |
| [9] | N. A. Eaves et al., "CoFlame: A refined and validated numerical algorithm for modeling sooting laminar coflow diffusion flames," Computer Physics Communications, vol. 207, 2016 |
| [10] | V. Chernov et al., "Numerical investigation of soot formation mechanisms in partially-premixed ethylene–air co-flow flames," Combustion and Flame, vol. 159, no. 9, 2012 |
| [11] | Ronneberger, O., Fischer, P., & Brox, T. (2015). U-Net: Convolutional Networks for Biomedical Image Segmentation. In Medical Image Computing and Computer-Assisted Intervention (MICCAI) |
| [12] | Yi Wu, Zhen Li, Qianlong Wang, Guillaume Legros, Chaomin Li, and Zhiwen Yan, "Predicting simultaneously fields of soot temperature and volume fraction in laminar sooting flames from soot radiation measurements - a convolutional neural networks approach," Opt. Express 30, 21230-21240 (2022) |
| [13] | A. Rodríguez et al., "Improving broadband emission-based soot pyrometry using convolutional neural networks," IEEE I2MTC, 2021 |
| [14] | Ren, Tao & Zhou, Ya & Wang, Qianlong & Liu, Haifeng & Li, Zhen & Zhao, Changying. (2021). Machine learning-assisted soot temperature and volume fraction fields predictions in the ethylene laminar diffusion flames. Optics Express. 29. 1678-1693. 10.1364/OE.413100 |
| [15] | Shizhan, Qiao, et al. "Flame Temperature Detection and Estimation Model Based on Deep Learning and Ordinary RGB Images." 2021 2nd International Conference on Artificial Intelligence and Information Systems. 2021 |
| [16] | Agrawal, Goutam, et al. "Flame Temperature Prediction Using Machine Learning Model." 2020 IEEE India Council International Subsections Conference (INDISCON). IEEE, 2020 |
| [17] | Dai, Minglu, et al. "3-D soot temperature and volume fraction reconstruction of afterburner flame via deep learning algorithms." Combustion and Flame 252 (2023): 112743 |
| [18] | K. He et al., "Deep residual learning for image recognition," in CVPR, 2016 |